PSDRIVER - 0.2 (Oct 17, 1993)

=============================

Abstract

--------

PSDRIVER.SYS is a DOS-Device-Driver that implements a printing

device for printing ASCII documents on a postscript printer.

This first release supports a subset of line drawing characters

and many language-specific characters of code page 437. It was

tested under MS-DOS 6.0, Banyan Vines 4.11 network and a Postscript

compatible Mannesmann Tally MT906PS printer.

This release is free, try it and report your experience to me.

For further enhancements, the author welcomes any comments. You

may contact me on Compuserve:

Johannes Geenen; Compuserve-ID: 100137,3124

Summary

-------

This DOS device driver allows direct output of ASCII files to a

postscript printer. This driver includes some extensions of the

IBM character set, including Umlaute (like �, � or �) and a set

of line drawing characters. Some basic Epson style escape

sequences are supported.

PSDRIVER has to be installed as a device driver in your

CONFIG.SYS file (may be loaded high) and occupies about 7K of

memory. On default, the driver replaces LPT3 and redirects it to

the first (BIOS-)printer port. Different output devices may be

specified on command line.

Installation

------------

Copy the driver onto your hard disk and add to your CONFIG.SYS file

the following statement (change the path, if you copied the

driver to a different location than the root directory):

DEVICE=C:\PSDRIVER.SYS

and reboot your computer. On startup PSDRIVER will display the

used memory segment, the device name used as primary (input)

device and the printer port used for output to the postscript

printer:

ASCII to POSTSCRIPT Translator 0.2

Installed at segment.: 03CA

Using device name....: LPT3

Printing to port.....: 1

The segment address depends on your additional configuration and

DOS version. Device name and printer port may be changed by

command line arguments. The complete command line syntax of

PSDRIVER is:

DEVICE[HIGH]=<Your Drive And Path>\PSDRIVER.SYS <Input> <Output>

where:

Your Drive And Path: Location of PSDRIVER.SYS on your

hard disk

Input              : Name of the device you want to

print your ASCII files to (LPTn,

PSDRV, etc.), Default: LPT3

Output             : Number of the printer port, the

postscript printer is attached to

(1, 2 or 3), Default: 1

This package contains two drivers: PSDRIVER.SYS and PSDRVLET.SYS.

The only difference between this drivers is, that PSDRIVER.SYS

assumes A4 paper size, while PSDRVLET.SYS assumes Letter paper size.

The PSDRVLET.SYS has not been tested.

Usage

-----

To test the driver, just copy a plain text file to the device,

that PSDRIVER has installed (or replaced):

COPY CONFIG.SYS LPT3

The text will be printed onto your postscript printer (if

attached to the output printer port and online, of course!);

unprintable characters will be replaced by an asterisk ('*'),

Backspace, Tab, Line- and Formfeeds and Carriage Return control

characters will be recognized and interpreted. A formfeed (print

the last page) is appended automatically.

Of course, you may use the original printer port, since

PSDRIVER does not modify it's behavior. Just have

in mind: Print a plain text file to the input device of PSDRIVER,

the driver will do the rest (hopefully).

Features

--------

* Prints ASCII files to your postscript printer without any menu

changes (PSDRIVER converts the text file to a Postscript file).

* Allows printing of national character sets and some IBM graphics

extensions.

* Supports some basic Epson style escape sequences (see below),

including Elite and Compressed pitch selection, character

emphazizing (bold, italic, underlined), super- and subscript,

setting of line density.

* Allows use on LAN work stations with a postscript network

printer (tested on our Banyan Vines 4.11 LAN).

Supported Epson Escape commands

-------------------------------

ESC 0       Select 8 LPI

ESC 2       Select 6 LPI (default)

ESC 4       Select italic font

ESC 5       Reset italic to upright

ESC E       Select bold face

ESC F       Reset bold face

ESC <SI>

<SI>        Set Compressed font (15 CPI)

<DC2>       Set Compressed off

ESC g       Set 15 CPI font

ESC M       Set Elite font (12 CPI)

ESC P       Set Pica font (10 CPI, default)

ESC - <1>   Underlining on

ESC - <0>   Underlining off

ESC S <0>   Superscript on

ESC S <1>   Subscript on

ESC T       Reset Super- & Subscript

ESC C <n>   Set page length to n lines

ESC 3 <n>   Set line feed to n/180 inch

ESC A <n>   Set line feed to n/60 inch

Future enhancements

-------------------

* Improving the command line interface

* Adding a user interface: change fonts, page orientation and

margins using an interface program.

* Support of virtually any character of the IBM-set

* Reducing the resident memory size

Technical Information

---------------------

PSDRIVER is a small device driver, which uses the DOS-built-in

character device driver interface for processing files. On an

open request the postscript header is downloaded to the printer,

then the file itself is downloaded (performing the neccessary

character translations) and on closing request a final form feed

is performed.

Most of the resident driver size is occupied by the postscript

header which is compressed using a very simple Huffman like

compression method. This reduces the header size from about

8K uncompressed to compressed 6K (this may be improved, I

know!). The header includes already all stuff for font selection,

font sizes, orientation, bolding or underlining and will

hopefully not grow very much during enhancement of functionality.

History

-------

I developed the PSDRIVER program, because I often need to print

program files out of my DOS based text editor. The first approach

was a small programm, that switched the emulation of the printer

between Postscript and Laserjet. But then two things happened:

A new postscript printer was installed (with a different

emulation switching code) and I was attached to a network printer

(Ever changed the emulation from Postscript to Laserjet on a

network printer? Just wait 'til the network administrator - or

angry users - come around...)

Version 0.1 (Sep 09, 1993)

Very first release, prints ASCII files with a Pica font

Version 0.2 (Oct 17, 1993)

Reduced resident size, added basic commands of ESC/P printer

control language (Epson FX)

The End

-------

So I decided to write this driver. Based on my small assembler

knowledge, Michael J. Young's book "Performance Programming Under

MS-DOS", Sybex 1987, and other stuff, this release of the device

driver was coded in three days or so.

Some enhancements I mentioned above, wait inside my little gray

brain cells to be coded, others are a bit more difficult to me:

* How could I access disk files during installation (for

configuration files)?

* How should I design the user interface (I want to use IOCTL

functions, but how should I determin the device name (or

handle), when not using the default ones?)

Warranty

========

You may use this program on any DOS-PC. I've tested it under DOS 6.0, but

it may work under DOS 3.X, too. Of course, the author can give no warranty

at all, including failures on hardware, software or damaging of data.

(There is no code in PSDRIVER, which may cause such defects, but who knows?).

The Author

==========

Mail:   Johannes Geenen

Schwannstrasse 21

D-41460 Neuss

Germany

E-Mail: Compuserve   100137,3124